TASK 4 — BILL OF MATERIALS (BOM) FOR AN RC AIRCRAFT

Electric, Single-Motor Fixed-Wing Aircraft

1. Aircraft Brief Requirements

| Parameter | Requirement Specification |
| --- | --- |
| Wingspan | 1.0 – 1.2 m |
| Configuration | High wing / Conventional fixed-wing configuration |
| Propulsion | Single electric brushless motor |
| Control | 4-Channel Wireless Remote Control (Throttle, Ailerons, Elevator, Rudder) |
| Structure | Lightweight, crash-resistant, and easily repairable (Foam / Balsa / Carbon Reinforcement) |
| Takeoff Mode | Hand launch or Smooth Ground Takeoff (Tricycle / Taildragger gear) |

2. Part A — Identified Components & Bill of Materials (BOM)

| Component | Selected Specification / Model | Quantity | Unit Cost (₹) | Total Cost (₹) |
| --- | --- | --- | --- | --- |
| Motor | SunnySky X2212 1400KV Brushless Motor | 1 | 1,200 | 1,200 |
| ESC | Flycolor 30A ESC with 5V/2A BEC | 1 | 650 | 650 |
| Propeller | 10x4.5 Slow-Fly 2-Blade Propeller | 1 pair | 250 | 250 |
| Battery | Orange 3S 2200mAh 30C LiPo Battery | 1 | 1,450 | 1,450 |
| Airframe | Expanded Polypropylene (EPP) Kit (1.1m) | 1 kit | 1,800 | 1,800 |
| Reinforcement | Carbon Fiber Rods 3mm OD x 1m | 2 | 150 | 300 |
| Servos | TowerPro SG90 9g Micro Servos | 4 | 150 | 600 |
| Transmitter & Rec. | FlySky FS-i6 6CH 2.4GHz + IA6B Receiver | 1 set | 4,200 | 4,200 |
| Landing Gear | Pre-bent Aluminum Main Gear + 2" Wheels | 1 set | 350 | 350 |
| Hardware & Supplies | Pushrods, Horns, CA Glue, XT60 Plug | 1 set | 400 | 400 |
| Total Estimated Build Cost |  |  |  | ₹11,200 |

3. Part B — Component Comparison & Trade-Off Options

| Component | Recommendation 1 | Recommendation 2 | Key Specifications / Comparison | Qty | Est. Cost (₹) |
| --- | --- | --- | --- | --- | --- |
| Motor | SunnySky X2212 1400KV | A2212 1000KV Brushless | KV rating, max rated current, prop compatibility | 1 | 1,200 |
| ESC | Flycolor 30A ESC | Yellow Jacket 30A ESC | Continuous current rating, BEC output (5V/2A) | 1 | 650 |
| Propeller | 10x4.5 SF Propeller (2-pack) | 9x6E Electric Propeller | Diameter x Pitch, adapter shaft size (3.17mm) | 1 pair | 250 |
| Battery | Orange 3S 2200mAh 30C | Bonka 3S 1500mAh 45C | Cell count (3S), capacity (mAh), C-rating | 1 | 1,450 |
| Airframe | Expanded Polypropylene (EPP) | Corrugated Plastic (Coroplast) | Wingspan, weight, flexibility & repairability | 1 kit | 1,800 |
| Reinforcement | Carbon Fiber Rod 3mm | Fiberglass Tape 25mm | Torsional stiffness, flexural rigidity, weight | 2 | 300 |
| Servos | TowerPro SG90 9g Servo | EMAX ES08A II 9g Servo | Torque (1.8 kg-cm), gear type, response speed | 4 | 600 |
| Transmitter & Rec. | FlySky FS-i6 + IA6B | Radiomaster Pocket CC2500 | 2.4GHz protocol, range, receiver channel count | 1 | 4,200 |
| Landing Gear | Aluminum Main + Tail Wheel | Carbon Fiber Fixed Gear | Ground clearance, impact resistance, mounting | 1 set | 350 |
| Hardware & Supplies | Pushrods, Horns, CA Glue | Hot Glue + Pushrod Connectors | Control linkage hardware, adhesive durability | 1 set | 400 |

4. Part C — Engineering Selection & Technical Justification

1. Motor–Propeller Match & Thrust Verification: A 1400KV motor with a 10x4.5 prop on 3S LiPo produces ~850g static thrust at 16A current draw. Thrust is verified using a digital thrust-stand setup measuring static force against current draw.

2. ESC Rating & Electrical Margin: Maximum continuous current draw from motor is ~16A–18A. Selecting a 30A ESC provides a safe ~40% electrical margin, preventing thermal shutdown and ensuring longevity.

3. Battery Selection & Trade-Offs: A 3S 2200mAh 30C LiPo offers an optimal balance between weight (185g) and flight time (~10–12 minutes). Higher capacity adds excessive mass without proportional runtime benefit.

4. Servos & Control Surfaces: 4 micro servos (9g each) assigned to: Aileron Left, Aileron Right (dual-servo wing setup), Elevator (pitch control), and Rudder (yaw / ground steering control).

5. Structural Material Selection: EPP foam is chosen for the airframe due to high impact resistance and simple field repairability using CA glue or hot glue. Carbon fiber spars provide essential flexural rigidity.

6. Electrical & Mechanical Compatibility: ESC BEC delivers 5V regulated power to the RX receiver and 4 servos via standard 3-pin DuPont headers. 2.4GHz AFHDS 2A protocol ensures robust frequency-hopping against interference.

5. Part D — Budget Analysis & Configuration Trade-Offs

1. Total Estimated Build Cost: Estimated complete build cost for the chosen setup is approximately ₹11,200 (including transmitter/receiver transmitter kit). Component-only build cost without TX/RX is ~₹7,000.

2. Safe Cost-Saving Opportunities: Reusing an existing transmitter/receiver saves ₹4,200. Utilizing EPP/Coroplast sheet DIY build instead of a pre-molded kit saves ~₹800 safely without sacrificing structural integrity.

3. Budget vs. Performance Configurations: Budget Configuration (₹8,500 total): A2212 motor, 30A non-branded ESC, 3S 1500mAh battery, Coroplast frame.
Performance Configuration (₹13,500 total): SunnySky X2212 motor, Flycolor 30A ESC, EPP pre-molded frame, Digital metal-gear servos.

4. Selected Competition Configuration & Defense: The standard balanced performance setup (₹11,200 complete) is selected. It guarantees high thrust-to-weight ratio, structural repairability, and reliable frequency-hopping link crucial for competition environment.

6. Bonus — Takeoff Mass & Propulsion Assessment

| Subsystem Component | Mass Estimate (g) |
| --- | --- |
| Airframe (EPP + Carbon Rods + Hardware) | 280 g |
| Propulsion (Motor + ESC + Propeller) | 110 g |
| Battery (3S 2200mAh LiPo) | 185 g |
| Electronics (4x Servos + Receiver + Wiring) | 65 g |
| Total All-Up Mass (AUW) | 640 g |

Propulsion Assessment: With an estimated Total Takeoff Mass (AUW) of 640g and available static thrust of 850g (from 1400KV motor on 10x4.5 prop), the resulting Thrust-to-Weight Ratio is 850g / 640g = 1.33:1. A ratio > 1.0 ensures effortless hand launch, rapid rate of climb, and agile recovery performance under competition conditions.